# RESIDENTIAL ZONING

The following is a summary of the area regulations for the City single-family zoning districts.  The complete residential zoning code and ARB Guidelines are available on the City’s website at www.glendalemo.org:

# R-1 Zoning Regulations (Generally)

Front Yard Setback:			35 feet (25 feet for unenclosed front porches)

Rear Yard Setback:			30 feet

Side Yard Setback:			10 feet

Lot Size Requirement:			10,000 sq. feet (min 80 feet wide)

Height:					35 feet or 2 ½ stories

Corner Lots

Front Yard Setback:			35 feet (Adjacent to each street)

Double Frontage Lots

Front Yard Setback:			35 feet	(Both front & rear)

Lots of Record

Front Yard (interior):	Distance based on the average of the front building lines of the two main buildings on either side.

Front Yard (corner lot):	Building line as delineated on a subdivision plat of record. Otherwise, distance on side street will be based on 25% of the width of the lot Min. 12 ft.

Rear Yard:				30 feet

Side Yard:				10% of the width of the lot

(Min. 6 feet to max. 10 feet)

Accessory Structures & Detached Structures

Front Yard:				Not Permitted

Rear Yard:				6 feet

Side Yard:				6 feet

Distance From House:			10 feet

Rear Yard Density:	No more than 30% of req. rear yard may be used for accessory buildings.

Deck Exception:	May be located as close as 15 feet from rear property line.

There are additional regulation in regard to the creation of new lots dealing with width and buildable width and overall square footage.

# R-2 Zoning Regulations (Generally)

Front Yard Setback:			30 feet (20 feet for unenclosed front porches)

Rear Yard Setback:			30 feet

Side Yard Setback:			7 feet (more than 65 feet in width)

6 feet (65 feet or less)

Lot Size Requirement:			7500 sq. feet (min 65 feet wide)					Height:					35 feet or 2 ½ stories

Corner Lots

Front Yard:				30 feet

Rear Yard:				30 feet

Side Yard:				7 feet (more than 65 feet in width)

6 feet (65 feet or less)

Double Frontage

Front Yard:				30 feet (Both front & rear)

Lots of Record

Front Yard (interior or corner):	Distance based on the average of the front building lines of the two main buildings on either side.

Front Yard (corner lot):	Building line as delineated on a subdivision plat of record. Otherwise, distance on side street will be based on 25% of the width of the lot. Min. 12 ft.

Rear Yard:				30 feet

Side Yard:				7 feet (more than 65 feet in width)

6 feet (65 feet or less)

Accessory Structures

Accessory rules for detached structures apply in both the R-1 and R-2 Zoning Single Family Zoning Districts.

# FLOOR AREA RATIO (FAR)

The FAR limit for all lots zoned R-1 or R-2 is 0.30. FAR is calculated by dividing the gross floor area of a house by the total area of the lot. Gross Floor Area includes all areas provided with heat and/or air conditioning. Includes all conditioned half stories with ceiling heights of more than 5 feet. All living space with ceiling heights of sixteen (16) feet or greater shall be counted at 200%. Attached garages shall be counted at 50%. Any finished or unfinished basement, detached garages, and unenclosed porches are excluded.